Electric Potential on a 2D Grid

This project simulates how electric potential behaves on a two-dimensional grid using NVIDIA GPU libraries. Electric potential describes how much energy a unit charge has at each point in space. Regions near a positive charge have high potential, and those near a negative charge have low potential.

The goal was to solve Poisson’s equation:

∇²φ = -ρ/ε₀

where φ is potential, ρ is charge density, and ε₀ (vacuum permittivity) controls how strongly charges interact. The program builds a grid and computes potential values when a single point charge is placed in the center.

The grid is represented as a system of linear equations Aφ = b, where A captures how each grid cell interacts with its four neighbors using a five-point stencil. The matrix is stored in a compact CSR (Compressed Sparse Row) format to save memory. The right-hand side vector b (the charge distribution) is generated directly on the GPU using Thrust, avoiding unnecessary data transfers.

To solve the system, cuSOLVER’s sparse Cholesky routine computes the potential efficiently. The result is verified by computing the residual with cuSPARSE, which checks how close Aφ is to b. A typical 64×64 simulation converges with an average residual around 10⁻⁶, showing strong numerical accuracy.

A major challenge was that the matrix can become singular which means the solver cannot find a unique solution—because potential is relative. This was solved by “pinning” one grid point to zero volts, providing a reference point. Another challenge was adapting to updated CUDA APIs: newer versions require cusparseSpMV instead of older deprecated functions. Finally, I verified that the charge sign affected potential direction; a positive charge produces a high potential (hill), while a negative charge forms a low potential (well).

The final version adds automatic CSV output and a heatmap generator, producing a visual plot of potential values (potential_<gride_size>.png). The center appears bright for a positive charge and dark for a negative one, matching physical expectations.

This project taught me how physical concepts like electric potential can be modeled through numerical methods, and how GPU acceleration (cuSOLVER, cuSPARSE, Thrust) makes these simulations much faster and more scalable.